BAO CAO PROJECT
THIET KE VA TRIEN KHAI MANG METRO ETHERNET SU DUNG MPLS
CHO KET NOI DA CHI NHANH DOANH NGHIEP

Mon hoc: Thiet ke mang

Cong cu trien khai: Ubuntu/Linux, Mininet, Open vSwitch, Python, iperf3, ping, traceroute

Ghi chu: Mat khau sudo khong duoc luu trong source code, log, README hay bao cao.

# Muc luc

1. Gioi thieu de tai

2. Muc tieu nghien cuu

3. Co so ly thuyet Metro Ethernet MAN

4. Co so ly thuyet MPLS

5. Thiet ke mo hinh mang

6. Kien truc Branch 1 Flat Network

7. Kien truc Branch 2 Three-tier Network

8. Kien truc Branch 3 Leaf-Spine Network

9. Thiet ke Provider MPLS Backbone

10. Vai tro CE, PE, P router

11. Trien khai tren Mininet

12. Cau hinh dinh tuyen/forwarding

13. Kiem thu ket noi thuc te

14. Do throughput, delay, packet loss, jitter

15. Bang ket qua thuc te

16. Bieu do so sanh hieu nang

17. Phan tich anh huong cua tung kien truc LAN

18. Danh gia hoat dong MPLS hoac forwarding tuong duong MPLS

19. So sanh MPLS voi IP routing truyen thong

20. Han che cua mo hinh

21. Ket luan

22. Tai lieu tham khao

# 1. Gioi thieu de tai

De tai xay dung mot mo hinh Metro Ethernet MAN ket noi 3 chi nhanh doanh nghiep qua ha tang nha cung cap. Topology duoc chay that bang Mininet; cac so lieu trong bao cao lay tu ping, iperf3 va traceroute trong luc topology dang hoat dong.

# 2. Muc tieu nghien cuu

Muc tieu la thiet ke mo hinh CE-PE-P, cau hinh Linux kernel MPLS lien chi nhanh, kiem thu connectivity va do throughput, delay, packet loss, jitter.

# 3. Co so ly thuyet Metro Ethernet MAN

Metro Ethernet la dich vu ket noi Ethernet trong pham vi do thi/khu vuc, cho phep doanh nghiep mo rong LAN qua mang nha cung cap voi toc do cao, quan tri tap trung va chi phi hop ly.

# 4. Co so ly thuyet MPLS

MPLS chuyen tiep goi dua tren label thay vi chi dua vao tra cuu IP hop-by-hop. Router bien co the push label, router loi swap label, router ra pop label truoc khi giao goi ve mang dich.

# 5. Thiet ke mo hinh mang

Mo hinh gom Branch 1 dang Flat Network, Branch 2 dang Three-tier, Branch 3 dang Leaf-Spine va provider backbone gom pe1, pe2, pe3, p1, p2, p3, p4.

Hinh 1. So do tong the topology.

# 6. Kien truc Branch 1 Flat Network

Branch 1 co 4 host noi vao 2 switch access b1_sw1/b1_sw2, sau do ket noi den CE router b1_ce.

Hinh 2. Branch 1 Flat Network.

# 7. Kien truc Branch 2 Three-tier Network

Branch 2 gom access, distribution, core va CE. Cach chia lop giup mo hinh de mo rong va de quan tri hon.

Hinh 3. Branch 2 Three-tier Network.

# 8. Kien truc Branch 3 Leaf-Spine Network

Branch 3 co 4 leaf, 2 spine va CE. Trong lab nay topology duoc giu loop-free de OVS khoi dong on dinh, tap trung vao kiem thu forwarding/MPLS thay vi hoi tu STP.

Hinh 4. Branch 3 Leaf-Spine Network.

# 9. Thiet ke Provider MPLS Backbone

Provider gom 3 PE router ket noi 3 CE va 4 P router mesh lam backbone. Luu luong lien chi nhanh bat buoc di qua PE/P.

Hinh 5. Provider Backbone.

# 10. Vai tro CE, PE, P router

CE nam phia khach hang, PE nam bien provider va ket noi CE vao backbone, P router nam trong loi provider va chuyen tiep luu luong giua cac PE.

# 11. Trien khai tren Mininet

Cac node router la Linux namespace bat IP forwarding. Cac LAN switch dung Open vSwitch standalone. File topology_mininet.py co CLI de demo thu cong.

## 11.1 Giao dien giam sat va demo truc quan

GUI Tkinter cho phep chon source/destination host va chay truc tiep cac phep do ping, traceroute, throughput, delay, packet loss va jitter. Cua so log rieng giup phong to/thu nho output lenh khi thuyet trinh.

Hinh 6. Giao dien GUI monitor dung de demo cac phep do.

# 12. Cau hinh dinh tuyen/forwarding

Mode MPLS su dung Linux kernel MPLS that: FRRouting OSPF/LDP tao route loopback PE/P va label dong; traffic CE lien chi nhanh di qua service VPLS/L2VPN. Data-plane L2VPN trong Mininet dung full-mesh Linux GRETAP pseudowire co split-horizon/port-isolation tren underlay MPLS de co the kiem chung bang ping va tcpdump. Bang chung cau hinh nam trong results/mpls_routes.txt va goi MPLS bat bang tcpdump nam trong results/tcpdump_mpls.txt.

## 12.1 Minh chung bang route MPLS kernel

Trich xuat bang route MPLS that tren PE/P. Cac dong `encap mpls`, `proto ldp` va `as to` the hien label switching dong trong backbone.

===== pe1: ip route show =====
2.2.2.2 nhid 34  encap mpls  22 via 172.20.13.2 dev pe1-eth2 proto ospf metric 20 
3.3.3.3 nhid 28  encap mpls  26 via 172.20.11.2 dev pe1-eth1 proto ospf metric 20 
11.11.11.11 nhid 29 via 172.20.11.2 dev pe1-eth1 proto ospf metric 20 
22.22.22.22 nhid 30  encap mpls  16 via 172.20.11.2 dev pe1-eth1 proto ospf metric 20 
33.33.33.33 nhid 35 via 172.20.13.2 dev pe1-eth2 proto ospf metric 20 
44.44.44.44 nhid 36  encap mpls  23 via 172.20.13.2 dev pe1-eth2 proto ospf metric 20 
172.20.11.0/30 dev pe1-eth1 proto kernel scope link src 172.20.11.1 
172.20.12.0/30 nhid 29 via 172.20.11.2 dev pe1-eth1 proto ospf metric 20 
172.20.13.0/30 dev pe1-eth2 proto kernel scope link src 172.20.13.1 
172.20.23.0/30 nhid 35 via 172.20.13.2 dev pe1-eth2 proto ospf metric 20 
172.20.24.0/30 nhid 37  encap mpls  24 via 172.20.13.2 dev pe1-eth2 proto ospf metric 20 
172.20.32.0/30 nhid 31  encap mpls  17 via 172.20.11.2 dev pe1-eth1 proto ospf metric 20 
172.20.34.0/30 nhid 33  encap mpls  25 via 172.20.11.2 dev pe1-eth1 proto ospf metric 20 

===== pe1: ip -f mpls route show =====
18 via inet 172.20.11.2 dev pe1-eth1 proto ldp 
19 as to 16 via inet 172.20.11.2 dev pe1-eth1 proto ldp 
20 via inet 172.20.13.2 dev pe1-eth2 proto ldp 
21 via inet 172.20.11.2 dev pe1-eth1 proto ldp 
22 via inet 172.20.13.2 dev pe1-eth2 proto ldp 
23 as to 17 via inet 172.20.11.2 dev pe1-eth1 proto ldp 
24 as to 22 via inet 172.20.13.2 dev pe1-eth2 proto ldp 
25 as to 23 via inet 172.20.13.2 dev pe1-eth2 proto ldp 
26 as to 24 via inet 172.20.13.2 dev pe1-eth2 proto ldp 
27 as to 25 via inet 172.20.11.2 dev pe1-eth1 proto ldp 
28 as to 26 via inet 172.20.11.2 dev pe1-eth1 proto ldp 

===== pe1: sysctl net.mpls.platform_labels =====
net.mpls.platform_labels = 100000

===== pe2: ip route show =====
1.1.1.1 nhid 27  encap mpls  16 via 172.20.23.2 dev pe2-eth1 proto ospf metric 20 
3.3.3.3 nhid 33  encap mpls  24 via 172.20.24.2 dev pe2-eth2 proto ospf metric 20

## 12.2 Minh chung bang tcpdump va hex dump

Tcpdump tren core link p3-eth0 bat duoc ethertype 0x8847 va MPLS shim header 4 byte; day la bang chung goi tin di bang label thay vi chi IP routing.

tcpdump: listening on p3-eth0, link-type EN10MB (Ethernet), snapshot length 262144 bytes
17:37:52.518122 96:cb:20:c1:bc:14 > 00:00:00:00:00:10, ethertype MPLS unicast (0x8847), length 116: MPLS (label 22, tc 0, [S], ttl 64)
	(tos 0x0, ttl 64, id 41432, offset 0, flags [DF], proto GRE (47), length 98)
    1.1.1.1 > 2.2.2.2: GREv0, Flags [key present], key=0x64, proto TEB (0x6558), length 78
	5a:71:7c:9a:7a:b0 > 33:33:00:00:00:02, ethertype IPv6 (0x86dd), length 70: (hlim 255, next-header ICMPv6 (58) payload length: 16) fe80::5871:7cff:fe9a:7ab0 > ff02::2: [icmp6 sum ok] ICMP6, router solicitation, length 16
	  source link-address option (1), length 8 (1): 5a:71:7c:9a:7a:b0
	    0x0000:  5a71 7c9a 7ab0
	0x0000:  0000 0000 0010 96cb 20c1 bc14 8847 0001  .............G..
	0x0010:  6140 4500 0062 a1d8 4000 402f 928f 0101  a@E..b..@.@/....
	0x0020:  0101 0202 0202 2000 6558 0000 0064 3333  ........eX...d33
	0x0030:  0000 0002 5a71 7c9a 7ab0 86dd 6000 0000  ....Zq|.z...`...
	0x0040:  0010 3aff fe80 0000 0000 0000 5871 7cff  ..:.........Xq|.
	0x0050:  fe9a 7ab0 ff02 0000 0000 0000 0000 0000  ..z.............
	0x0060:  0000 0002 8500 dbb5 0000 0000 0101 5a71  ..............Zq
	0x0070:  7c9a 7ab0                                |.z.
17:37:52.518198 96:cb:20:c1:bc:14 > 00:00:00:00:00:10, ethertype MPLS unicast (0x8847), length 116: MPLS (label 22, tc 0, [S], ttl 64)
	(tos 0x0, ttl 64, id 41433, offset 0, flags [DF], proto GRE (47), length 98)
    1.1.1.1 > 2.2.2.2: GREv0, Flags [key present], key=0x64, proto TEB (0x6558), length 78
	00:00:00:00:00:12 > 33:33:00:00:00:02, ethertype IPv6 (0x86dd), length 70: (hlim 255, next-header ICMPv6 (58) payload length: 16) fe80::200:ff:fe00:12 > ff02::2: [icmp6 sum ok] ICMP6, router solicitation, length 16
	  source link-address option (1), length 8 (1): 00:00:00:00:00:12
	    0x0000:  0000 0000 0012
	0x0000:  0000 0000 0010 96cb 20c1 bc14 8847 0001  .............G..
	0x0010:  6140 4500 0062 a1d9 4000 402f 928e 0101  a@E..b..@.@/....
	0x0020:  0101 0202 0202 2000 6558 0000 0064 3333  ........eX...d33
	0x0030:  0000 0002 0000 0000 0012 86dd 6000 0000  ............`...
	0x0040:  0010 3aff fe80 0000 0000 0000 0200 00ff  ..:.............
	0x0050:  fe00 0012 ff02 0000 0000 0000 0000 0000  ................
	0x0060:  0000 0002 8500 7b0a 0000 0000 0101 0000  ......{.........
	0x0070:  0000 0012                                ....
17:37:52.800865 96:cb:20:c1:bc:14 > 00:00:00:00:00:10, ethertype MPLS unicast (0x8847), length 144: MPLS (label 22, tc 0, [S], ttl 64)
	(tos 0x0, ttl 64, id 41552, offset 0, flags [DF], proto GRE (47), length 126)
    1.1.1.1 > 2.2.2.2: GREv0, Flags [key present], key=0x64, proto TEB (0x6558), length 106

# 13. Kiem thu ket noi thuc te

Kiem thu dung ping giua Branch1-Branch2, Branch1-Branch3 va Branch2-Branch3. Log chi tiet nam trong results/ping_results.txt.

# 14. Do throughput, delay, packet loss, jitter

Throughput do bang iperf3 TCP, jitter co ban do bang iperf3 UDP 10M, delay do bang ping, duong di goi tin do bang traceroute. Packet loss khi tai mang tang duoc do bang iperf3 UDP voi cac muc tai 5M, 20M, 50M, 80M va 120M.

# 15. Bang ket qua thuc te

Thoi gian kiem thu tu results.csv: 2026-05-06T17:37:58 den 2026-05-06T17:41:10.

Bang 1 gom cac phep do baseline giua tung cap host dai dien.

| timestamp | mode | test_type | load_mbps | source_branch | destination_branch | source_host | destination_host | throughput_mbps | avg_delay_ms | packet_loss_percent | jitter_ms | udp_packet_loss_percent | note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2026-05-06T17:37:58 | mpls | baseline |  | Branch1 | Branch2 | host1 | user1 | 0.208 | 33.447 | 0.0 | 0.11 | 0.0 | success |
| 2026-05-06T17:38:15 | mpls | baseline |  | Branch1 | Branch3 | host2 | svr1 | 0.208 | 40.148 | 0.0 | 0.106 | 0.0 | success |
| 2026-05-06T17:38:33 | mpls | baseline |  | Branch2 | Branch3 | user1 | svr2 | 0.208 | 35.491 | 0.0 | 0.126 | 0.0 | success |
| 2026-05-06T17:38:50 | mpls | baseline |  | Branch1 | Branch1 | host1 | host4 | 94.713 | 6.559 | 0.0 | 0.081 | 0.0 | success |
| 2026-05-06T17:39:08 | mpls | baseline |  | Branch2 | Branch2 | admin1 | guest1 | 93.964 | 13.019 | 0.0 | 0.124 | 0.0 | success |
| 2026-05-06T17:39:26 | mpls | baseline |  | Branch3 | Branch3 | svr1 | os1 | 94.302 | 10.83 | 0.0 | 0.069 | 0.0 | success |

## 15.1 Bang packet loss theo muc tai

Bang 2 ghi ket qua UDP packet loss sweep khi tang offered load tren cac cap lien chi nhanh.

| timestamp | mode | test_type | load_mbps | source_branch | destination_branch | source_host | destination_host | throughput_mbps | avg_delay_ms | packet_loss_percent | jitter_ms | udp_packet_loss_percent | note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2026-05-06T17:39:49 | mpls | udp_load_sweep | 5.0 | Branch1 | Branch2 | host1 | user1 |  |  |  | 0.14 | 0.0 | success |
| 2026-05-06T17:39:54 | mpls | udp_load_sweep | 20.0 | Branch1 | Branch2 | host1 | user1 |  |  |  | 0.149 | 0.0 | success |
| 2026-05-06T17:39:59 | mpls | udp_load_sweep | 50.0 | Branch1 | Branch2 | host1 | user1 |  |  |  | 0.261 | 0.0 | success |
| 2026-05-06T17:40:05 | mpls | udp_load_sweep | 80.0 | Branch1 | Branch2 | host1 | user1 |  |  |  | 0.366 | 22.933 | success |
| 2026-05-06T17:40:11 | mpls | udp_load_sweep | 120.0 | Branch1 | Branch2 | host1 | user1 |  |  |  | 0.284 | 52.429 | success |
| 2026-05-06T17:40:16 | mpls | udp_load_sweep | 5.0 | Branch1 | Branch3 | host2 | svr1 |  |  |  | 0.093 | 0.0 | success |
| 2026-05-06T17:40:22 | mpls | udp_load_sweep | 20.0 | Branch1 | Branch3 | host2 | svr1 |  |  |  | 0.077 | 0.0 | success |
| 2026-05-06T17:40:27 | mpls | udp_load_sweep | 50.0 | Branch1 | Branch3 | host2 | svr1 |  |  |  | 0.3 | 1.894 | success |
| 2026-05-06T17:40:33 | mpls | udp_load_sweep | 80.0 | Branch1 | Branch3 | host2 | svr1 |  |  |  | 0.254 | 30.834 | success |
| 2026-05-06T17:40:38 | mpls | udp_load_sweep | 120.0 | Branch1 | Branch3 | host2 | svr1 |  |  |  | 0.263 | 48.983 | success |
| 2026-05-06T17:40:44 | mpls | udp_load_sweep | 5.0 | Branch2 | Branch3 | user1 | svr2 |  |  |  | 0.134 | 0.0 | success |
| 2026-05-06T17:40:49 | mpls | udp_load_sweep | 20.0 | Branch2 | Branch3 | user1 | svr2 |  |  |  | 0.086 | 0.0 | success |
| 2026-05-06T17:40:54 | mpls | udp_load_sweep | 50.0 | Branch2 | Branch3 | user1 | svr2 |  |  |  | 0.241 | 0.0 | success |
| 2026-05-06T17:41:00 | mpls | udp_load_sweep | 80.0 | Branch2 | Branch3 | user1 | svr2 |  |  |  | 0.283 | 22.372 | success |
| 2026-05-06T17:41:10 | mpls | udp_load_sweep | 120.0 | Branch2 | Branch3 | user1 | svr2 |  |  |  | 0.247 | 52.892 | success |

# 16. Bieu do so sanh hieu nang

Hinh 7. Throughput.

Hinh 8. Delay trung binh.

Hinh 9. UDP packet loss khi tai mang tang.

Hinh 10. Jitter.

# 17. Phan tich anh huong cua tung kien truc LAN

Flat Network co duong LAN ngan nen de cau hinh. Three-tier co them access/distribution/core nen so hop noi bo nhieu hon. Leaf-Spine tao cau truc hien dai hon cho data center, nhung trong mo hinh nay duong di van bi anh huong boi backbone provider.

# 18. Danh gia hoat dong MPLS native

Project da cau hinh Linux kernel MPLS native trong cac namespace PE/P. FRRouting OSPF/LDP tao bang route co `proto ospf`, `proto ldp`, `encap mpls` va `show mpls table`. Backbone PE/P dong thoi chay FRRouting OSPF/LDP; frr_control_plane.txt co OSPF Full, LDP OPERATIONAL, label binding va show mpls table. Luong CE/branch di qua service VPLS/L2VPN; data-plane L2VPN trong Mininet duoc hien thuc bang full-mesh Linux GRETAP pseudowire co split-horizon/port-isolation tren underlay MPLS va co bridge/gretap state trong frr_control_plane.txt.

# 19. So sanh MPLS voi IP routing truyen thong

MPLS trong mang that co uu diem ve traffic engineering, VPN va chuyen tiep theo label. Static IP routing de trien khai trong lab nhung kem linh hoat khi mang lon va nhieu chinh sach dich vu.

# 20. Han che cua mo hinh

Lab chay tren mot may ao nen ket qua phu thuoc CPU, kernel, OVS va tai he thong. MPLS native yeu cau kernel co module mpls_router va mpls_iptunnel.

# 21. Ket luan

Project da trien khai duoc topology Metro Ethernet da chi nhanh, cau hinh forwarding, kiem thu lien chi nhanh va tao bao cao dua tren so lieu that.

# 22. Tai lieu tham khao

Mininet Documentation

Open vSwitch Documentation

iperf3 User Documentation

RFC 3031 - Multiprotocol Label Switching Architecture

Metro Ethernet Forum - Ethernet Services Overview